Dispersion Relation

sinω∆t2= ± vp∆t1∆x2sin2kx∆x2+1∆y2sin2ky∆y2

Phase Velocity

vp= ωk= ±2k∆tsin-1vp∆t1∆x2sin2kcosθ∆x2+1∆y2sin2ksinθ∆y2

If ∆x= ∆y

vp= ωk= ±2k∆tsin-1vp∆t∆xsin2kcosθ∆x2+sin2ksinθ∆y2

Which poses the restriction

vp∆t∆x≤ 12→maxsin2kcosθ∆x2=1

Application of Sources

Hard Source

Ey|i,jn+1= Ey|i,jn- ∆tε∆xHz|i+12,jn+12- Hz|i-12,jn+12 + Ey,source|i,jn+1

Hard Current Source

∂E∂tn+12= 1ε∇×Hn+12- 1εJsourcen+12

Where the source current is located at the same spatial grid location as the electric field but at the same time point as the magnetic field. Which is nearly the same as the Hard Source if we assume that εEy,source= ∆tJsource

Interface Condition

Hzi+12,j

Hzi-12,j

Hyi,j+12

Exi,j

Hyi,j-12

Hzi+12,j-Hzi-12,j∆y+Hyi,j+12-Hyi,j-12∆z= ∂∂t εo+ ε12 Ei,j